# Technical data

This chapter outlines values and information that restrict intended use.

The units of measurement are metric.

The technical data in the following documents must also be observed:

- “Specific data and KHS standards”

- “Electrical circuit diagram”

- “Purchased parts documentation”

- “Integrated components”

  - Media requirements

Information about the requirements for media can be found in the technical documentation “Specific data and KHS standards”, chapter “K0255 — Media quality”.

    - Electrical energy

Requirements: see “Electrical specifications”.

  - Operation

    - Machine data

| Data | Value | Unit |
| --- | --- | --- |
| Ambient temperature | 18-35 | °C |
| Maximum relative humidity | 85 | % |

    - Supply data

| Data | Value | Unit |
| --- | --- | --- |
| Electrical voltage | 400 | V |
| On-site backup fuse | 35 | A |

    - Noise emissions

      - Information on noise emissions

| Operating conditions and specific noise-related characteristics | Nominal capacity: see chapter “Technical data” The noise emission levels may increase or decrease according to changes in operating condi‐ tions or specific characteristics. |
| --- | --- |
| Specified noise emission level dyads in accordance with ISO 4871 |  |
| A-weighted emission sound pressure level LpA in dB at work‐ places KpA uncertainty | 75-82 +/- 2 |
| C-weighted peak emission sound pressure level LpCpeak in dB at workplaces KpCpeak uncertainty | Does not exceed 130 +/- 2 |
| A-weighted acoustic power LwA in dB KwA uncertainty | Not specified due to very large machine dimensions ./. |
| Basic standards EN ISO 11204 or EN ISO 3744, accuracy class 2 were used to determine the levels in accordance with EN 415-9. |  |
| Emission data sheets are available on request containing tables showing the noise emissions at each of the measurement points and a sketch of the machine indicating the locations of these meas‐ urement points. |  |

      - Noise Reduction/Acoustics Requirements

The machines to be delivered comply with advanced noise reduction techniques tried and tested in the beverage industry in effect at the time of order placement.

The emission levels at the workstations of the line are not only dependent on the emissions of the particular machines but also on the interrelationship of the sources of noise within the room layout, outside noises (neighboring systems, ventilation, etc.), and the room's acoustics.

A line comprised of several machines equipped with preferably con‐ trolled, interconnecting conveying facilities is able to comply with advanced noise reduction techniques tried and tested in practical operation only if the future operator makes low-noise dimensioning

of machines and conveying segments possible and provides a sound- insulated room.

Room sound proofing is achieved by installing an effective sound- absorbing ceiling (high-pitch sound absorbers) in conjunction with absorbing wall paneling (particularly behind the operators).

Key room acoustics parameters are the reverberation time and the average decrease in sound intensity. According to the state of the art, the reverberation period must be 0.5 to 0.8 seconds in octave bands of between 500 Hz and 4 kHz, depending on the size of the room, and the average decrease in sound intensity should amount to at least 4.5 dB(A) each time the distance is doubled.

The emission sound level at the workstation is measured in dB(A) at a distance of 1 meter from the outside edge of the machine and at

a height of 1.6 meters from the floor when the operator is standing (ear height). If an operating face is installed (e.g. in front of the filler or bottle washer), the arithmetic average measured at 3 measurement points along the operating face applies for the sound level at the workstation. Floor spaces limited towards the back (e.g. platforms or catwalks) must therefore be at least 1.3 meters wide.

If the future operating company intends to provide machines or line components himself, he must ensure that these components also comply with the advanced rules of noise reduction and safety tech‐ nology tried and tested in practice.

Project planning shall be carried out chiefly with regard to the required line capacity, installed on the floor space provided by the customer.

It may therefore also be necessary to take additional secondary noise-reduction measures even after the acoustic requirements to be met on the part of the operator have been implemented.

    - Types

Container types

| Container no. | Name | Contents [l] | Diameter [mm] | Height [mm] |
| --- | --- | --- | --- | --- |
| 1 | BN33RE | 0.33 | 61.5 +/- 1.4 | 227.0 +/- 1.4 |
| 2 | Green Ale | 0.33 | 60.6 +/- 1.0 | 230.0 +/- 1.0 |
| 3 | CSD | 0.33 | 63.1 +/- 1.4 | 227.9 +/- 1.5 |
| 4 | Green Ale | 0.50 | 69.6 +/- 1.4 | 265.0 +/- 1.5 |
| 5 | Green Bremen | 0.65 | 76.5 +/- 1.4 | 280.0 +/- 1.4 |
| 6 | Green Appo | 0.65 | 74.9 +/- 1.4 | 282.5 +/- 1.7 |
| 7 | CSD | 0.65 | 74.7 +/- 1.5 | 289.0 +/- 1.5 |

Pack type

| Pack no. | Formation | Length [mm] | Width [mm] | Height [mm] | Used with container no. |
| --- | --- | --- | --- | --- | --- |
| 1 | 6x(6x4) | 445 | 298 | 283 | 1, 2, 3 |
| 2 | 10x(3x4) | 370 | 270 | 325 | 4, 5, 6, 7 |

Format part sets

| Format part set no. | Formation | Used with container no. |
| --- | --- | --- |
| 1 | 6x(6x4) | 1, 2, 3 |
| 2 | 10x(3x4) | 4 |
| 3 | 10x(3x4) | 5, 6, 7 |

    - Settings

      - Pneumatics

| Specification | Value | Unit |
| --- | --- | --- |
| Minimum operating pressure required | 5 | bar |
| Maximum operating pressure | 6 | bar |

Incorrect pneumatic equipment settings lead to machine damage during production.

    - Labeling of interchangeable parts

All interchangeable parts for a type are uniquely labeled. The label has two lines and is structured as follows:

| A | . | B | C | D | E |
| --- | --- | --- | --- | --- | --- |
| F |  |  |  |  |  |

- - Line designation according to customer specifications

- - Machine type code number

C - Machine type according to KHS — see “Appendix”, section

“Machine type”

D - System component — for interchangeable parts, always a value of 9100

- - Type designation according to customer specifications

- - Machine-specific

| A1030 | . | 1 | PU | 9100 | :06 |
| --- | --- | --- | --- | --- | --- |
| 4.1PE9100:01/02/03 |  |  |  |  |  |

Example

A1030.1PU9100:06 4.1PE9100:01/02/03

A1030 - Line designation according to customer specifications

1	- 1. machine of this machine type in the line PU	- Packer

9100	- Interchangeable part

:06	- Type designation according to customer specifications

4.	- Line designation in accordance with customer specifica‐ tions

1	- 1. Machine of this machine type in the line PU	- Packer

9100	- Interchangeable part

01/02/03 - Type designation according to customer specifications

  - Maintenance

    - Lubricants

NOTICE!

Wrong lubricant

Selecting the wrong lubricant causes damage to machine components.

–	Use only the lubricants as indicated in the lubrica‐ tion plan.

Information about the lubricants can be found in the technical docu‐ mentation “Specific data and KHS standards”, chapter “K1000 — Lubricant overview”.

    - Detergent

Information about the limits for detergents can be found in the tech‐ nical documentation “Specific data and KHS standards”, chapter “K0255 - Media quality”.

    - Labeling of equipment for disconnecting and securing

All equipment for disconnecting and securing is labeled in accordance with KHS standards.

Information about the labels can be found in the technical documenta‐ tion “Specific data and KHS standards”, chapter “K0357 - Operating material labeling”.