- Technical data

This chapter describes values and information that limit the intended use.

The units of measure are metric.

The technical data in the following documents must also be observed:

- ‘Specific data and KHS standards’

- ‘Remote access operating instructions’

- ‘Electrical circuit diagram’

- ‘Purchased parts documentation’

  - Media requirements

Information about the requirements for media can be found in the technical documentation ‘Specific data and KHS standards’, chapter ‘K0255 — Media quality’.

    - Compressed air

Compressed air must be provided in sufficient quantities and at the required pressure without fluctuations in quantity and pressure.

Compressed air quality classes

| Classification numbers | 1st number |  |  | 2nd number*) | 3rd number |
| --- | --- | --- | --- | --- | --- |
|  | Maximum number of particles/m3; particle size d (µm) |  |  |  |  |
| Class | 0.1 < d ≤ 0.5 | 0.5 < d ≤ 0.1 | 1 < d ≤ 5 | Pressure dew point (°C)*) | Residual oil content including mist (mg/m3) |
| 0 | Specified according to application and better than Class 1 |  |  |  |  |
| 1 | 100 | 1 | 0 | ≤ -70 | ≤ 0.01 |
| 2 | 100,000 | 1,000 | 100 | ≤ -40 | ≤ 0.1 |
| 3 | - | 10,000 | 500 | ≤ -20 | ≤ 1 |
| 4 | - | - | 1,000 | ≤ +3 | ≤ 5 |
| 5 | - | - | 20,000 | ≤ +7 | - |
| Quality classes according to DIN standard ISO 8573-1:2001 |  |  |  |  |  |

*) The pressure dew point must be at least 20°C lower than the coldest possible ambient temperature.

      - Compressed air [5.5-8 bar(a)]

This compressed air is used as control air for valves and pneumatic components.

| Characteristic | Value | Unit |
| --- | --- | --- |
| Quality according to DIN ISO 8573-1 | 2- 1*)-1 - Further information on page 431 |  |
| Pressure | 5.5-8 | bar(a) |
| Smell/taste | Odourless and tasteless | - |
| Temperature | 5-30 | °C |

  - Operation

    - Machine data Identification

| Data | Value |
| --- | --- |
| Type | NMP-H |
| Year of construction | 2024 |
| Machine number | 10354698 89409041-000400 |

Capacity

| Data | Value | Unit |
| --- | --- | --- |
| Nominal capacity | 108000 | Containers/h |
| Maximum number of cycles | 450 | Cycles/min |

Ambient conditions

| Data | Value | Unit |
| --- | --- | --- |
| Minimum ambient temperature | 10 | °C |

| Data | Value | Unit |
| --- | --- | --- |
| Maximum ambient temperature | 35 | °C |
| Maximum relative humidity | 70 | % |

Dimensions and weight of the packing machine

| Data | Value | Unit |
| --- | --- | --- |
| Length | 23700 | mm |
| Width | 2720 | mm |
| Height | 2600 +/- 50 | mm |
| Net weight | Approx. 25885 | kg |

Working height

| Data | Value | Unit |
| --- | --- | --- |
| Infeed | 1250 +/- 50 | mm |
| Outfeed | 1250 +/- 50 | mm |

Electrical connection – front machine section

| Data | Value | Unit |
| --- | --- | --- |
| Network configuration | 3/PE/N |  |
| Frequency | 50 | Hz |
| Operating voltage | 400 | VAC |
| Connected load | 82 | kVA |
| Nominal current | 118 | A |
| Control voltage | 24 | VDC |
| Maximum pre-fusing, supply line | 125 | A |
| Maximum voltage fluctuation | +/- 10 | % |
| Maximum frequency variation | +/- 2 | % |

Electrical connection – rear machine section

| Data | Value | Unit |
| --- | --- | --- |
| Network configuration | 3/PE/N |  |

| Data | Value | Unit |
| --- | --- | --- |
| Frequency | 50 | Hz |
| Operating voltage | 400 | VAC |
| Connected load | 77 | kVA |
| Nominal current | 110 | A |
| Control voltage | 24 | VDC |
| Maximum pre-fusing, supply line | 125 | A |
| Maximum voltage fluctuation | +/- 10 | % |
| Maximum frequency variation | +/- 2 | % |

Pneumatic connector – front machine section

| Data | Value | Unit |
| --- | --- | --- |
| Connection | ½ | Inch |
| Connection | 13 | mm |
| Minimum air pressure | 5.5 | bar |
| Minimum air pressure | 80 | psi |
| Maximum air pressure | 8 | bar |
| Maximum air pressure | 114 | psi |
| Operating air pressure | 6 | bar |
| Operating air pressure | 87 | psi |
| Air consumption | 25 | m3/h |

Pneumatic connector – rear machine section

| Data | Value | Unit |
| --- | --- | --- |
| Connection | ½ | Inch |
| Connection | 13 | mm |
| Minimum air pressure | 5.5 | bar |
| Minimum air pressure | 80 | psi |
| Maximum air pressure | 8 | bar |
| Maximum air pressure | 114 | psi |
| Operating air pressure | 6 | bar |
| Operating air pressure | 87 | psi |

| Data | Value | Unit |
| --- | --- | --- |
| Air consumption | 25 | m3/h |

    - Noise emissions

      - Information on noise emissions

| Operating conditions and specific noise-related characteristics | Nominal output: see Technical data Noise emission values can increase or decrease with changes in the operating condi‐ tions or specific characteristics. |
| --- | --- |
| Specified noise emission level dyads in accordance with ISO 4871 |  |
| A-weighted emissions sound pressure level LpA in dB at work‐ places KpA uncertainty | 75–82 +/- 2 |
| C-weighted peak emissions sound pressure level LpCpeak in dB at workplaces KpCpeak uncertainty | Does not exceed 130 +/- 2 |
| A-weighted acoustic power LwA in dB KwA uncertainty | Not specified due to very large machine dimensions ./. |
| The values were determined according to EN 415-9 using the EN ISO 11204 and/or EN ISO 3744 base standards, accuracy class 2. |  |
| Emission data sheets are available on request containing tables showing the noise emissions at each of the measurement points and a sketch of the machine indicating the locations of these meas‐ urement points. |  |

      - Noise reduction/acoustics requirements

The machines to be delivered comply with advanced noise reduction techniques that are tried and tested in the beverage industry and are in effect at the time of order placement.

The emission levels at the system workstations are not only dependent on the emissions of the relevant machines, but also on the interrelationship of the noise sources within the room layout, outside noises (neighbouring systems, ventilation, etc.), and the acoustics of the room.

A system comprising several machines equipped with preferably con‐ trolled, interconnecting conveyor units is only able to comply with

the tried-and-tested regulations for noise reduction techniques in effect at the time of order placement if the future operator enables low-noise dimensioning for machines and conveyors and provides a sound-insulated room.

Room sound insulation is achieved through the installation of an effective sound-absorbing ceiling (high-pitch sound absorbers) in con‐ junction with absorbing wall panelling (particularly behind the opera‐ tors).

Key room acoustics parameters are the reverberation time and the average decrease in sound intensity. According to the state of the art, the reverberation period must be 0.5 to 0.8 seconds in octave bands of between 500 Hz and 4 kHz, depending on the size of the room, and the average decrease in sound intensity should amount to at least 4.5 dB(A) each time the distance is doubled.

The emissions sound pressure level at the workstation is measured in dB(A) at a distance of 1 metre from the outside edge of the machine and at a height of 1.6 metres from the floor space where the operator is standing (ear height). If an operating face is installed (e.g. in front of the filler or cleaning machine), the arithmetic average measured

at three measurement points along the operating face applies to the sound level at the workstation. Floor spaces limited towards the back (e.g. platforms or catwalks) must therefore be at least 1.3 metres wide.

If the future operating company intends to provide machines or line components itself, it must ensure that these components also comply with the advanced rules of noise reduction and safety technology tried and tested in practice.

Project planning shall be carried out chiefly with regard to the required line capacity, installed on the floor space provided by the customer.

It may therefore also be necessary to take additional secondary noise-reduction measures even after the acoustic requirements to be met on the part of the operating company have been implemented.

    - Settings

The setting values for the adjustment points are included in the accompanying adjustment points list.

  - Maintenance

    - Lubricants

NOTICE!

Incorrect lubricant

Using an incorrect lubricant will damage the compo‐ nents in the machine.

–	Use only lubricants with the designation specified in the lubrication plan.

Information about the lubricants can be found in the technical docu‐ mentation ‘Specific data and KHS standards’, chapter ‘K1000 — Lubricant overview’.

Information about the labels can be found in the technical documenta‐ tion ‘Specific data and KHS standards’, chapter ‘K0357 — Operating material labelling’.

    - Detergent

Information about the limits for detergents can be found in the tech‐ nical documentation ‘Specific data and KHS standards’, chapter ‘K0255 — Media quality’.

      - Limit values for surface cleaning

NOTICE!

Material damage due to acidic chemicals

Improper use of acidic chemicals will cause material damage.

- Always be certain to avoid strong mineral acids such as HCl (hydrogen chloride), H2SO4 (sulphuric acid), and H2SO3 (sulphurous acid).

- Adhere to the application concentration levels, tem‐ peratures and periods specified by the manufac‐ turer.

- Avoid localised increases in concentration caused by the formation of droplets or evaporation.

- Avoid disinfection by soaking, e.g. overnight.

- After applying acidic media, ensure that surfaces are thoroughly rinsed with water.

NOTICE!

Reaction of stainless steel with cleaning chemicals

Exposure of stainless steel surfaces to cleaning chem‐ icals containing chlorine, chlorides, halogens and sulphates (combined with chlorides). The effect is increased in combination with high temperatures, high concentrations and low pH.

This could result in corrosion, which may cause product flaws, operational downtime and damage or destruction of components.

- Adhere to the requirements of the media.

- The exposure time must not exceed 20 minutes.

- Expose components to only cold chlorinated deter‐ gents.

- Rinse surfaces off thoroughly after cleaning with water and then dry. The chloride content of the fresh water used for rinsing must not exceed 50 ppm (parts per million).

NOTICE!

Oxygen-releasing detergents

Oxygen-releasing detergents corrode sealing mate‐ rials.

Do not use these detergents in every cleaning cycle.

NOTICE!

Penetration of detergent

If the lift cylinders are not pressurised during cleaning, detergent will penetrate the lift cylinders. This will result in a reduction of the service life of the seals used. A further negative effect is increased corrosion.

–	Before cleaning the machine surfaces, open the air supply to the lift cylinders.

NOTICE!

Detergent residue

If detergent residues are not removed by rinsing, evaporating water content will increase the concentra‐ tion levels of acids and caustics on stainless steel and plastic surfaces. As a result, surfaces may be destroyed by corrosion.

- Remove detergent residue from the machine sur‐ faces by rinsing them with drinking water.

- Use drinking water with a chloride content that does not exceed 150 ppm (parts per million).

- Re-rinse with water until normal methods of anal‐ ysis find no traces of the detergent used.

- Rub polished machine parts dry or blow-dry them with compressed air.

NOTICE!

Chlorides in detergents

Chlorides in detergents can cause pitting.

–	Do not exceed the maximum concentration of chlor‐ ides in detergents. The maximum recommended chloride content values are listed in the tables below.

All of the chemical agents offered by well-known companies can generally be used for cleaning the machines. Although KHS uses only high-quality materials in the manufacture of its machines, the manufacturer of the detergents should be consulted to determine the corrosiveness of the chemicals to be used. Some manufacturers offer clearance certificates, which should be sought in all instances.

The use of alkaline and neutral cleaning agents and disinfectants is usually sufficient. If acidic media are to be used, the user must take particular care in order to prevent material damage.

Contact the KHS laboratory in special cases and/or if in doubt.

        - Limit values table for detergents Source: KHS standard K0255 (05.2014)

Nitric acid (HNO3)/phosphoric acid (H3PO4)

| Requirement | Value |
| --- | --- |
| Concentration | < 1% |
| Form | In aqueous solution |
| Contact duration | < 30 min |
| Temperature | 25°C (room temperature) |
| Total hardness | < 20°dH |
| Chloride | < 20 mg/l |
| Iron | < 0.05 mg/l |
| Manganese | < 0.05 mg/l |

Sodium hydroxide solution for cleaning in place (NaOH)

| Requirement | Value |
| --- | --- |
| Concentration | < 2.5 % |
| Form | In aqueous solution |
| Contact duration | < 30 min |
| Temperature | < 80°C |
| Chloride | < 30 mg/l |
| Iron | < 0.05 mg/l |
| Manganese | < 0.05 mg/l |

Cleaning boosters (active oxygen based)

| Requirement | Value |
| --- | --- |
| Concentration | < 2 % |
| Form | In alkaline solution |
| Contact duration | < 30 min |
| Temperature | < 80°C |
| Chloride | < 30 mg/l |
| Iron | < 0.05 mg/l |
| Manganese | < 0.05 mg/l |

Combined alkaline cleaning solution (NaOH; hypochlorite)

| Requirement | Value |
| --- | --- |
| Concentration | < 2% (pH > 11) |
| Contact duration | < 45 min |
| Temperature | < 70 °C |
| Iron | < 0.05 mg/l |
| Manganese | < 0.05 mg/l |
| Sodium hypochlorite (NaClO) |  |
| Requirement | Value |
| Concentration | 15 mg/l |
| Form | In aqueous solution |
| Contact duration | < 20 min |
| Temperature | < 30 °C |
| pH value | < 7 |
| Chloride | < 50 mg/l |
| Iron | < 0.05 mg/l |
| Manganese | < 0.05 mg/l |
| Peracetic acid (C2H4O3) |  |
| Requirement | Value |
| Concentration | < 500 mg/l |
| Contact duration | < 15 min |
| Temperature | < 20 °C |
| Chloride | < 30 mg/l |

Iodophor-based disinfection by soaking

| Requirement | Value |
| --- | --- |
| Concentration (active iodine in acidic product) | < 50 mg/l |
| Contact duration | < 16 h |
| Temperature | < 30 °C |
| Chloride | < 30 mg/l |

Sanitisation with hot water for conventional applications

| Requirement | Value |
| --- | --- |
| Contact duration | < 45 min |
| Temperature | 85-95°C |

Chlorine dioxide (ClO2)

| Requirement | Area of applicability | Value |
| --- | --- | --- |
| Concentration | In open systems | < 2 mg/l |
|  | In closed and continually circulated systems | 5 mg/l |
| Temperature |  | < 30 °C |
| Iron and/or man‐ ganese |  | 0.1 mg/l |

Ozone (O3)

| Requirement | Value |
| --- | --- |
| Concentration | 0.8 - 2 mg/l |
| pH value | 6-7 (closer to pH 6) |
| Temperature | 10-20°C |
| Water hardness | < 4°dH |
| Anolyte |  |
| Requirement | Value |
| Free chlorine content | < 4 mg/l |
| Chloride | < 150 mg/l |
| pH value | 6.5-7.5 |
| Temperature | 20 °C |
| Carbonate hardness of mixed water | < 5°dH |

Catholyte

| Requirement | Value |
| --- | --- |
| Sodium hydroxide (NaOH) | 4 % |
| Chloride | < 100 mg/l (at pH < 10) |
| pH value | > 10 |
| Temperature | 20 °C |
| Carbonate hardness of mixed water | < 5°dH |

    - Labelling of equipment for disconnecting and securing

All equipment for disconnecting and securing is labelled in accord‐ ance with KHS standards.

Information about the labels can be found in the technical documenta‐ tion ‘Specific data and KHS standards’, chapter ‘K0357 — Operating material labelling’.

  - Certificates

    - EC declaration of conformity

KHS hereby declares that the supplied machine conforms to the applicable harmonised standards and will affix the CE marking to the machine. The operator receives the EC declaration of conformity.

CE labelling

Figure 212: CE labelling

The CE labelling is located on the name plate. See chapter ‘General information’, section ‘Machine assignment’.

The following illustration shows the content of the EC declaration of conformity. The operator receives the original document with date and signature as an individual document.

Translation

EC/EU Declaration of Conformity

According to Machinery Directive 2006/42/EC, OJ L 157, 09.06.2006, p. 24, Annex II, 1A

Manufacturer:	KHS GmbH – Juchostraße 20 – 44143 Dortmund – Germany

This declaration of conformity is issued under the sole responsibility of the manufacturer

Person authorized to compile the technical files:

Mr Schmidt, Volker – Boschstraße 1-3 – 47533 Kleve, Germany

Machine description / identification:

The designated machine is an industrial equipment that is installed fixed at their installation site. The machine contains integrated equipment for the purpose of radio communication or radiodetermination.

Machine designation:	Sammelpackmaschine

Function:	Bildung von Sammelpackungen

Model/ type:	NatureMultiPacker

Serial number:	10354698 89409041-000400

Order no.:	89409041

Commercial name:	KHS Innopack Kisters NMP-H

Installation site:	Carlsberg UK Ltd. – 140 Bridge St. – NN1 1PZ Northampton – GB

We hereby declare that the machine specified above fulfills all the relevant provisions of the Machinery Directive 2006/42/EC.

The machine also is in conformity with the following relevant EU harmonisation legislation:

| harmonized standards according to Art. 7, Sec. 2 | other standards and technical specifications |
| --- | --- |
| EN ISO 12100:2010 | ETSI EG 203 367 V1.1.1 (2016-06) |
| EN ISO 13849-1:2015 |  |
| EN ISO 13849-2:2012 |  |
| EN 60204-1:2018 |  |
| EN 61000-6-2:2005/AC:2005 |  |
| EN 61000-6-4:2007/A1:2011 |  |
| EN 415-7: 2006+A1:2008 |  |

Radio Equipment		2014/53/EU, OJ L 153, 22.05.2014, p. 62 Reference to applied …

Signed for and on behalf of KHS GmbH:

Place/date:

Kleve / 05.01.2024

Last name, first name, position:

________________________________

Brücker, Peter, Leiter Produkt Center Verpackungstechnik

Signature

Q-FB 4 (englisch), Stand: 13.06.2017	Page 1 of 1

This page is left blank for typographical reasons.